Pick To Light XML Sequence Configuration

This document explains the sequence.xml file for Pick To Light (P2L). It explains everything, including the steps intended for T&A engineers.

XML files are used to configure a P2L sequence, in other words they define the steps that are executed. The files are stored in \\veris-fs01\Production Line Data\[line]\[station]\Configs\XML.

XML Sections:

<powerup>:

All steps in the powerup section will run as soon as the program launches. They must finish running before an operator can start a sequence.

<scan> / <persist>:

Persist steps run at the end of the P2L sequence regardless of model and regardless of passing or failing. If something fails and the sequence must stop, these steps will run before the operator is allowed to start a new sequence. These steps also run when the reset button is pressed. They are typically used to turn off all lights, or in the case of testers, turn off all power and enter a safe state.

<scan> / <init>:

These are the first steps to run after initial scan. They run for all models. This is a good place to do database reads and determine which model sequence to run.

<scan> / <sequence model=”something”>

The main sequence for each part. All steps within this sequence will only execute if the scanned part matches the “model” attribute. The model attribute can take any regex or exact value. In this screenshot it is equal to “test”, meaning the steps inside will run for the part number “test” and no other (technically anything containing “test” within it). Model is compared against the “modelnumber” variable (see the variables section a few pages down).

<scan> / <sequence> / <persist>:

These persist steps are similar to the one above but they only run for this specific model. They will execute before the main persist above.

<scan> / <sequence> / <section>:

The main guts. Steps can be divided into smaller sections if desired, or the whole sequence can run as one big section. Put the vast majority of steps here. Currently the <section> tag has no functionality, but this may be added in the future.

<buttonX>:

In this case, X is a number 1-7

These steps execute when a button is pressed. Button 1 is in the top right, button 7 in the bottom right. Buttons can be enabled/disabled and their names customized in the config.ini file. All button tasks run asynchronous to the main sequence. They will have access to the main sequence’s variables. Remember, variables are cleared upon starting a new sequence

<maintenance>:

In this case, X is a number 1-4

These steps will run when the corresponding maintenance button is pressed. The maintenance buttons can be found by expanding the P2L window using the button with arrows next to “Exit Program”.

Execution:

After operator scan, the sections will execute in this order:

- <scan> / <init>

- <scan> / <sequence model=””> / <section>

- <scan> / <sequence model=””> / <persist>

- <scan> / <persist>

XML Step Explanation:

This line is one step that the P2L program will execute.

It contains 3 parameters:

- Name, with a value of “set_light”

- Description, with a value of “Turning on light #1”

- Light, with a value of “1”

The name parameter describes the name of the step being executed. In this case the step being executed is called “set_light”, which in the P2L code is programmed to turn on a light. Some parameters are unique to specific step names, while others are global (can be used on all steps)

There are more possible parameters here, but not all are required. Parameters that are not required have default values. Below is a list of all global and step unique parameters, what they do, and their default values.

Variables:

Variables are used to allow steps to “communicate” with one another. Step #1 can save a variable that step #2 can then read and/or modify. The variables that are created during a sequence can be viewed using the “step mode” switch or through the log file.

Some variables are special:

- Scan

- This variable contains the value that the operator scanned to start the sequence

- Modelnumber:

- This variable is compared against the <sequence model=””> section in the XML. If this variable is never set in the “init” section, the sequence may fail to run

- To get this set quickly, try doing the following:

- Serialnumber:

- This variable is used in the name of the log file

Global Parameters:

These parameters can be used on all steps (all possible step names). Starred parameters (*) are required. Others are optional and will resort to their default values if not provided

Name*:

- Expects a string

- Determines the name of the step to run. These names should match a case in the “Run XML Step.vi” VI.

Stoponfail:

- Expects “true” or “false”

- If this parameter is set to “true” and the step fails, the sequence will immediately run its persist steps and stop.

- Default value: True. Uses this value if unexpected input is given (i.e. spelling)

Model:

- Expects a string or regular expression

- Set this parameter to a model number or regular expression. Model numbers contained in this expression will run this step. The rest will skip over it.

- Default value: Blank. All model numbers will execute this step

Description:

- Expects a string

- The description that is shown in the description box, and eventually the completed box. If the description is not provided or is left blank, P2L will run the step without updating the description or competed boxes, essentially hiding the fact that it’s running.

- Default value: Blank.

Async:

- Expects “true” or “false”

- Set this to true to run steps in the background. Only 1 step can be ran in the background at a time, in other words an async step will run only after previous async steps have finished. The completed box will show this is running in the background (provided the description is not left blank). If an asynchronous step fails and stoponfail is true, a message will be displayed saying a background task failed and the foreground step will stop as soon as possible.

- Default value: false. Uses this value if unexpected input is given

Skipvar:

- Expects a string

- Set this to the name of a variable. If that variable is found and is a value of “true”, this step will be skipped.

- Default value: Blank. This step will not be skipped

Runvar:

- Expects a string

- Set this to the name of a variable. If that variable is found and is a value of “true”, this step will run, otherwise it will be skipped

- Default value: Blank. This step will run

Passvar:

- Expects a string

- The name of a variable whose value is set to “true” if this step passes

- Default value: Blank. No variable is set by default

Failvar:

- Expects a string

- The name of a variable whose value is set to “true” if this step fails

- Default value: Blank. No variable is set by default

Step Names and Individual Parameters

NOT UP TO DATE

Description of all (most) steps and their unique parameters. A steps “name” comes from the name parameter described in the global parameters list above. A list of names can be found by examining the case statement in the VI “Run XML Step.vi”.

Starred parameters (*) are required. Failure to provide these parameters will cause the step to not properly execute. Others are optional and will resort to their default values if not provided

If a step has required variables, this means those variables must exist before calling the step, otherwise it may fail or return undesired results

Accusine_cleco_tool_info:

Baa_create_device:

- Unique to the BAA cell (MPC/MPV/ASP)

- Generates a BAA serial number and creates a parent device in the Odin database. Inserts a variable called “deviceid” that contains the device’s ID in the database

- Parameters:

- Modelprefix*:

- Expects a 3 to 4 character string

- This must match a value found in the table “tblDeviceTypes” in the Odin database. It is required to create a parent device

- Required variables:

- Modelnumber

Baa_get_sn_from_scan:

- Extracts the serial number from a scan and determines if a scan is for overpack. The variable “overpack” is created and set to “true” if the “scan” variable is “OVERPACK”

- Parameters:

- Regex:

- Expects a regular expression

- The part of the scan that should be the serial number

- Default value: .*

- Removedash:

- Expects “true” or “false”

- Removes the first dash that appears. This step is executed on the result of the above regular expression

- Default value: true

- Required variables:

- scan

Baa_make_datamatrix_label_vars:

- Creates the datamatrix string and a serial number with a – for later comparison. Stores this in the variable “datamatrix”. Also creates a variable “serialwithdash”.

- Parameters:

- Beginningstring:

- Expects a string

- The text to be inserted at the beginning of the data matrix string. Entered as a parameter because it’s unknown if it will remain constant

- Required variables:

- Serialnumber

Baa_make_prod_label_vars:

- Gathers the information for the product label from the database and writes it into different variables. These variables are called “productdatamatrix”, “hardwareversion”, “softwareversion”, “macaddress”, “qr_url”.

- Parameters:

- Customerpartnumber:

- Expects a string

- The customer part number that is put in the label. For example, MP-V-7A-BAA. Because BAA has to be complicated and different

- Ouibank:

- Expects an integer

- This refers to the MAC address OUI bank used in the Odin stored procedure spGetMACAddress

- Urlprefix:

- Expects a string

- The beginning of the URL for the QR code. This is concatenated with the serial number and saved to the qr_url variable. For example, if set to www.google.com, and the SN is 12345, qr_url will become www.google.com12345

- Required variables:

- serialnumber

backflush:

- Backflushes a part through the VerisMfg web service

- Parameters:

- Partnumbervar:

- Expects a variable name

- The name of the variable that contains the part number to backflush. This is the part that is being created

- Default value: modelnumber

- Quantity:

- Expects a number

- Quantity of parts to backflush

- Default value: 1

- Logfilepath:

- Expects a filepath

- Where the backflush logs shall be stored. These logs are all in a single file and make it easier for bulk backflush traceability

- Default value: Empty. In this case, no logs are saved

Banner_close:

- Closes the modbus reference opened by banner_init

- Parameters: None

Banner_init:

- Opens a modbus reference to the Banner DXMR90 light controller. All banner steps are written with the assumption that the DXMR90 is running the “uselight.xml” configuration file. If this step is generating errors, make sure the communication is allowed through the trellix firewall

- Parementers:

- Ip*:

- Expects an IP address

- This is the IP address of the controller to connect to

- Port:

- Expects an integer

- The modbus TCP port to connect through

- Default value: 502

Banner_set_light:

Banner_draft_light:

Banner_set_all_lights:

Delay:

- A step that runs and does nothing for a specified amount of time

- Parameters:

- Delay*:

- Expects an integer in milliseconds

- The length of time to delay

- Default value: 1000

Description_set:

Display_image:

- Displays an image next to the description in the HMI

- Parameters:

- Imagepath*:

- Expects a file path

- The full (not relative) path to the image to display

Ean_code_lookup:

Hipot_test:

Keyence_iv3_vision_test:

lds_eperform_send_part:

- Sends a part number to LDS and increases the good or bad part count by 1. Part numbers are sent to STS:10:64, good parts to 4X:1, bad parts to 4X:5

- Parameters:

- Partnumbervar:

- Expects a variable name

- The name of the variable that has the part number to send

- Default value: modelnumber

- Badpartreasoncode:

- Expects a number

- The LDS Bad Part Reason Code. Sent to modbus register 4X:60

- Default value: 0

- Quantity:

- Expects a number

- The number of good or bad parts to send

- Default value: 1

- Bad:

- Expects “true” or “false”

- Set this to true to send these parts as bad rather than good

- Default value: false

Lsa_pra_get_firmware:

Message_box_one_button:

Message_box_one_button_image:

Message_box_two_button:

meters_commit_test_record:

- Commits a test record to the meters database through the Save Test function of the TestDB web service

- Parameters:

- Applicationname*:

- Expects a string

- The ApplicationName parameter for the web service

- Testcategoryid:

- Expects an integer

- The TestCategory_ID parameter for the web service

- Default value: 0

- Required variables:

- Modelnumber

- Serialnumber

- Starttime

- Created in the meters_init_test_record step

meters_get_pn_from_pcba_sn:

- Returns the product serial number when given the PCBA/supplier serial number

- Parameters:

- Pcbasnvariable:

- Expects the name of a variable

- The name of the variable containing the PCBA serial number

- Default value: pcbaserialnumber

meters_init_test_record:

- Creates a variable called starttime used by the meters_commit_test_record step

- Parameters: None

meters_set_process_step:

- Sets the process step in the meters database for interlocking

- Parameters:

- Testidvar:

- Expects the name of a variable

- The name of the variable containing the test ID

- Default value: testid

- Step*:

- Expects an integer

- The step number

meters_test_passed_check:

- Checks the meters database to see if the previous product test passed (had a result code of 0)

- Parameters

- Modelnumbervar:

- Expects the name of a variable

- The variable that contains the model number to check

- Default value: modelnumber

- Serialnumbervar:

- Expects the name of a variable

- The variable that contains the serial number to check

- Default value: serialnumber

modbus_read:

- Reads a single modbus TCP register.

- Parameters:

- Ip*:

- Expects an IP address

- The IP address of the modbus unit to connect to and read from

- Port:

- Expects an integer

- The port to use for modbus connection

- Default value: 502

- Register*:

- Expects an integer

- The 0 indexed modbus register to read

- Variable:

- Expects the name of a new variable

- The variable to save the modbus result to

- Default value: modbusread

modbus_wait:

- Continuously reads a single modbus TCP register and waits in this step until that value matches the expected

- Parameters:

- Ip*:

- Expects an IP address

- The IP address of the modbus unit to connect to and read from

- Port:

- Expects an integer

- The port to use for modbus connection

- Default value: 502

- Register*:

- Expects an integer

- The 0 indexed modbus register to read

- Expected*:

- Expects an integer or a binary number beginning with 0b

- The value that this step is waiting to see. If this parameter is an integer, this step will wait until the modbus register value equals this integer. If this parameter begins with 0b, it is assumed to be a 16-bit binary number with 0’s, 1’s and X’s. This step will convert the modbus register value to binary and compare its bits to all non-X expected bits.

nop:

- Does nothing. Primarily used as a placeholder

Odin_create_link_child:

- Prompts for a user to scan a PCBA barcode, uses regex to sort out the model and serial numbers, creates the child device in the Odin database and links it to the parent device. If the child already exists, only updates its model and parent linkage

- Parameters:

- Modelregex:

- Expects a regular expression

- This expression is meant to return only the characters scanned that contain the model number

- Default value: .*

- Serialregex:

- Expects a regular expression

- This expression is meant to return only the characters scanned that contain the serial number

- Default value: .*

- Modelprefix*:

- Expects a 3 to 4 character string

- This must match a value found in the table “tblDeviceTypes” in the Odin database. It is required to create a parent device

- Deviceidvar:

- Expects the name of a variable

- This variable will store the newly created child device’s ID

- Default value: childdeviceid

- Modelnumbervar:

- Expects the name of a variable

- This variable will store the model number of the child device, i.e. whatever is the result of the modelregex parameter

- Default value: childmodelnumber

- Serialnumbervar:

- Expects the name of a variable

- This variable will store the serial number of the child device, i.e. whatever is the result of the serialregex parameter

- Default value: childserialnumber

- Expectedmodel:

- Expects a string

- The model number that this part is expected to have. A “scan is incorrect” message will be displayed if this value and the result of the modelregex parameter are not equal

- Default value: Not found. Nothing will be checked and no message box will be displayed

- Required variables:

- deviceid

Odin_create_veris_device:

odin_get_pn_from_sn:

- Gets a model number from a serial number in the Odin database. Calls the stored procedure “spGetModelNumberFromSerialNumber”

- Parameters:

- Serialnumbervar:

- Expects the name of a variable

- The variable containing the serial number to look up

- Default value: serialnumber

odin_test_passed_check:

- Gets the most recent test record of a product. Verifies that test passed on the correct tester within the timeframe

- Parameters:

- Serialnumbervar:

- Expects the name of a variable

- The name of the variable containing the serial number to check

- Default value: serialnumber

- Ischild:

- Expects “true” or “false”

- Specifies if the serial number passed in is a child (PCBA) serial number or not

- Default value: false

- Testsetids*:

- Expects a comma delimited list of numbers

- This is a list of testers to check for. TestSetIDs are stored in tblTestSets. For example, 178 is the ID of the LSA Environmental Tester

- Daystocheck:

- Expects an integer

- The maximum number of allowable days since the last passed test

- Default value: 9999999

ohaus_weigh:

ohaus_wait_zero:

Printvar:

- Prints the value of a variable. Great for debugging

- Parameters:

- Variable*:

- Expects a string

- The name of the variable to print

- Message:

- Expects a string

- A message to print alongside the variable. Place a “%s” in this string where the variable should be

- Default value: %s

Print_thermo_label:

- Prints a label using the veris Thermo label system. Calls the “PrintLabelObject” function in the ZPLLabelService web service.

- Parameters:

- Serialnumbervar:

- Expects a string

- The name of the variable containing the serial number

- Default value: serialnumber

- Modelnumbervar:

- Expects a string

- The name of the variable containing the model number

- Default value: modelnumber

- Firmwarevar:

- Expects a string

- The name of the variable containing the firmware version. This variable does not have to exist for some labels

- Default value: firmwareversion

- Datecodevar:

- Expects a string

- The name of the variable containing the date code. If this parameter is empty and the datecode variable is not found, P2L will get the current 4 digit datecode and use that value

- Default value: datecode

- Labeltype*:

- Expects an integer

- The label type. Find this information in Thermo

- Line*:

- Expects an integer

- The line number. Find this information in Thermo

- Printcount:

- Expects an integer

- The number of labels to print. All labels will contain the exact same information

- Default value: 1

- Names:

- Expects a comma delimited list. For example, “hardwareversion,softwareversion,macaddress”

- A list of custom names for Thermo. These are sent to the “Names” part of the “NameValuePair” input. All custom label information is populated here. Required if the values parameter is populated

- Default value: Blank

- Values:

- Expects a comma delimited list. Any item in that list beginning and ending in brackets [] is assumed to be a variable name and will be converted to that variable’s value. For example, “1.0.0,[softwareversion],00:00:00:00:00:00”.

- A list of custom values for Thermo. These are sent to the “Values” part of the “NameValuePair” input. All custom label information is populated here. Required if the names parameter is populated

- Default value: Blank

- Required variables:

- Serialnumber (if serialnumbervar parameter not provided)

- Modelnumber (if modelnumbervar parameter not provided)

- Firmwareversion (if firmwarevar parameter not provided)

- Any variables referenced in the parameters

Read_dio:

- Used with a NI IO Module (i.e. NI 6525, 6501, etc). Reads the modules inputs and waits for them to be an expected value (if applicable)

- Parameters:

- Taskname*:

- Expects an NI MAX task name (string)

- Tells the NI controller which task to execute. Tasks are created in the NI MAX program

- Expected:

- Expects “true” or “false”

- Causes this step to wait until the expected value is read in from the controller. If not found, step will read input and move on regardless of value

- Default value: Not Found

- Resultvar:

- Expects the name of a variable

- Variable to store the digital input’s value so it can be referenced in later steps. If not found, the input is not stored in any variable

- Default value: Not Found

Renamevar:

- Used to copy a variables value to a new variable.

- Parameters:

- Variable

- Expects the name of an existing variable

- The variable to be copied

- Newvariable

- Expects the name of a new or existing variable

- The variable to copy to. If the variable does not exist, it will be created and its data populated with old variable’s value. If the variable does exist, its value will be overwritten with old variable’s value.

- Regex:

- Expects a regular expression

- A regular expression that is performed on the value of the existing variable. The output of this expression is stored in the new variable

- Default value: .*

Scan_box_display:

Scan_check_constant:

- Brings up an HMI asking for an operators to scan something. Passes if the scan matches the expected value

- Parameters:

- Expected*:

- Expects a string or regular expression

- The text that is expected to find. This value is passed into a match regular expression function and returns true if there is any match. The scan will be marked as incorrect if there is no match

- Retry:

- Expects “true” or “false”

- Defines whether the user is allowed multiple attempts. Set to true to allow unlimited attempts, false to allow only one

- Default value: true

- This step also contains steps for Keyence scanners:

- Port (COMx)

- Timeout (integer in ms)

- Baud (integer)

- Databits (integer)

- Parity (none, off, even, space, mark)

- Stopbits (1, 2)

- Bank (01, 02, 03, etc)

- Timeout (amount of time before the scanner stops looking)

Scan_check_variable:

- Brings up an HMI asking for an operators to scan something. Passes if the scan matches the value of the expected value

- Parameters:

- regex:

- Expects a regular expression

- A regular expression to filter out the scanned value. Only the text that matches will be compared to the variable.

- Default value: Blank. The whole scan is compared.

- Variable*:

- Expects the name of a variable

- The name of the variable to compare to. Variables are compared exact; there is no regular expression support here

- Retry:

- Expects “true” or “false”

- Defines whether the user is allowed multiple attempts. Set to true to allow unlimited attempts, false to allow only one

- Default value: true

- This step also contains steps for Keyence scanners:

- Port (COMx)

- Timeout (integer in ms)

- Baud (integer)

- Databits (integer)

- Parity (none, off, even, space, mark)

- Stopbits (1, 2)

- Bank (01, 02, 03, etc)

- Timeout (amount of time before the scanner stops looking)

Scan_set_variable:

- Brings up an HMI asking for an operators to scan something. Passes if the scan matches the regular expression and saves the match to a variable

- Parameters:

- regex:

- Expects a regular expression

- A regular expression to filter out the scanned value. Only the text that matches this expression will be compared in the “expected” parameter and saved to the variable

- Default value: Blank. The whole scan is saved and no error checking occurs

- Expected:

- Expects a string or regular expression

- A regular expression to compare the output of the “regex” parameter to confirm it is what is expected. This will cause a “scan is incorrect” message if there is no match

- Default value: Blank. “scan is incorrect” shows up only if the output of “regex” is an empty string

- Variable*:

- Expects the name of a variable

- The name of the variable to save the result

- Retry:

- Expects “true” or “false”

- Defines whether the user is allowed multiple attempts. Set to true to allow unlimited attempts, false to allow only one

- Default value: true

- This step also contains steps for Keyence scanners:

- Port (COMx)

- Timeout (integer in ms)

- Baud (integer)

- Databits (integer)

- Parity (none, off, even, space, mark)

- Stopbits (1, 2)

- Bank (01, 02, 03, etc)

- Timeout (amount of time before the scanner stops looking)

Setvar:

- Sets the value of a variable

- Parameters:

- Variable*:

- Expects the name of a variable

- The variable to save to

- Value*:

- Expects a string

- The value to save

Trendpoint_link_second_sn:

Trendpoint_order_number:

Trendpoint_serialize:

Validate_variable_string:

Veris_get_sn_info:

Veris_request_sn:

Wait_for_async:

- Waits until all async steps have finished executing

- Parameters: None

Xuph001_close:

- Closes the modbus connection to a XUPH001 light controller. Ideally, run this in the “quit” section

- Parameters: None

Xuph001_init:

- Initializes the XUPH001 light controller. Creates a modbus TCP master connection with the device. Saves that connection reference in a global variable

- Parameters:

- Ip*:

- Expects a string

- IP address of the controller to connect to

- Port:

- Expects an integer

- The modbus TCP port to connect with

- Default value: 502

- Lightstate:

- Expects “steady” or “alternate”

- All lights will turn on either steady or will alternate. This cannot be done per individual light

- Default value: steady

- Ledcontrol:

- Expects “true” or “false”

- If this value is true, lights will turn off if they sense hands. If this is false, they will stay on until told to turn off

- Default value: true

- Temporization:

- Expects “0.25”, “0.5”, “1”, or “2”

- How long a light will stay on for after seeing a hand. Lower values can sometimes cause glitches. Higher values will make the P2L program seem slow

- Default value: 1

Xuph001_set_all_lights:

- Turns on or off all lights connected to a XUPH001 controller

- Parameters:

- State:

- Expected: “on” or “off”

- Defines if the lights should be turned on or off

- Default value: off

- Color:

- Expected: “green” or “red”

- The color of the lights

- Default value: green

Xuph001_set_light:

- Used with a XUPH001 light controller. Tells it which light to turn on

- Parameters:

- Light*:

- Expects an integer, 1 through 64

- Tells the controller which light to turn on.

- State:

- Expects “on” or “off”

- Toggles the light either on or off

- Default value: on

- Waitforhand:

- Expects “true” or “false”

- Determines if this step waits for a hand before moving on

- Default value: true

- Delay:

- Expects and integer in milliseconds

- Determines how long the P2L program should wait between the end of this step and the beginning of the next

- Default value: 300. Values below 300 are automatically converted to 300 to avoid glitches

Xuph001_teach_lights:

- Used with an XUPH001 light controller. Gives the ability to train the lights on which is which

- Parameters: None