Pick To Light XML Sequence Configuration

This document explains parts of the sequence.xml file for Pick To Light. It explains only the parts that an ME would typically handle.

It is recommended to open the XML file with Notepad++. This can be downloaded from the software center.

In the XML file, ME’s should only be concerned with one area, and it is between the 2 highlighted lines in the below screenshot:

Here is a larger screenshot of the area of concern. It is between the two <section> lines.

This contains the guts of the pick to light sequence. Each line represents one step in the sequence, and the steps are executed top to bottom.

XML Step Explanation:

This line is one step that the P2L program will execute.

It contains 3 parameters:

- Name, with a value of “set_light”

- Description, with a value of “Turning on light #1”

- Light, with a value of “1”

There are more possible parameters here, but not all are required. Parameters that are not required have default values. Below is a list of all global and step unique parameters, what they do, and their default values.

Any step beginning with <!-- and ending with --> is a comment. These are ignored by pick to light, so they are used to make the XML file more human-readable. An example of a comment:

Global Parameters:

These parameters can be used on all steps (all possible step names).

Starred parameters (*) are required. Others are optional and will resort to their default values if not provided.

Name*:

- Expects a string

- Determines the name of the step to run. This should match one of the Step Names listed in the Step Names and Individual Parameters below

Model:

- Expects a string or regular expression

- Set this parameter to a model number or regular expression. Model numbers contained in this expression will run this step. The rest will skip over it. For a very basic tutorial on regular expressions, see this guide (coming soon).

- Default value: Blank. All model numbers will execute this step

Description:

- Expects a string

- The description that is shown in the description box, and eventually the completed box. If the description is not provided or is left blank, P2L will run the step without updating the description or competed boxes, essentially hiding the step.

- Default value: Blank.

Step Names and Individual Parameters

Below is a description of all steps that an ME should handle and their unique parameters. A steps “name” comes from the name parameter described in the global parameters list above.

Starred parameters (*) are required. Failure to provide these parameters will cause the step to not properly execute. Others are optional and will resort to their default values if not provided.

NOTE: Not all steps are listed here. Only the ones that a non-T&A engineer is expected to deal with. If you wish to see all steps, take a look at the sequence guide for T&A engineers.

Banner_set_light:

- Used with a Banner DXMR90 light controller, I.e. the Banner branded pick to lights. Tells the controller which light to turn on. Lights should have labels on their top, and the label should contain a number 1-255.

- Parameters:

  - Light*:

    - Expects an integer, 1 through 255

    - Tells the controller which light to turn on.

  - State:

    - Expects “on” or “off”

    - Toggles the light either on or off. 99% of the time this should be on

    - Default value: on

  - Waitforhand:

    - Expects “true” or “false”

    - Determines if this step waits for a hand before moving on. 99% of the time this should be true

    - Default value: true

  - Delay:

    - Expects and integer in milliseconds

    - Determines how long the P2L program should wait between the end of this step and the beginning of the next

    - Default value: 0

Display_image:

- Displays an image next to the description in the HMI

- Parameters:

  - Imagepath*:

    - Expects a file path. The image must be a .png file

    - This is the full (not relative) path to the image to display. Leave this parameter blank to clear the image and go back to just the white.

Print_thermo_label:

- Prints a label using the veris Thermo label system.

- Parameters:

  - Labeltype*:

    - Expects an integer

    - The label type. Find this information in Thermo

  - Line*:

    - Expects an integer

    - The line number. Find this information in Thermo

  - Printcount:

    - Expects an integer

    - The quantity of labels to print. Note that all labels will contain the exact same information

    - Default value: 1

Scan_check_constant:

- Brings up an HMI asking for an operators to scan something. Passes if the scan matches the expected value

- Parameters:

  - Expected*:

    - Expects a string or regular expression

    - The text that is expected to find. This value is passed into a match regular expression function and returns true if there is any match. The scan will be marked as incorrect if there is no match

  - Retry:

    - Expects “true” or “false”

    - Defines whether the user is allowed multiple attempts. Set to true to allow unlimited attempts, false to allow only one

    - Default value: true

Xuph001_set_light:

- Used with a Telemecanique XUPH001 light controller, I.e. the Schneider branded pick to lights. Tells the controller which light to turn on. Lights should have labels on their top, and the label should contain a number 1-64.

- Parameters:

  - Light*:

    - Expects an integer, 1 through 64, OR a comma delimited list if turning on multiple lights, I.e. 1,2,3,4

    - Tells the controller which light to turn on (or off).

  - State:

    - Expects “on” or “off”

    - Toggles the light either on or off. 99% of the time this should be on

    - Default value: on

  - Waitforhand:

    - Expects “true” or “false”

    - Determines if this step waits for a hand before moving on. 99% of the time this should be true

    - Default value: true

  - Delay:

    - Expects and integer in milliseconds

    - Determines how long the P2L program should wait between the end of this step and the beginning of the next

    - Default value: 300. Values below 300 are automatically converted to 300 to avoid glitches

A quick example of the step from earlier with all of its parameters:

Here we are calling the step “xuph001_set_light”. Our description is “Light number 1   Z123456”, which is the text that will be displayed on the screen while the light is lit. We have a parameter called light, and it’s set to “1”, which tells the controller to turn on light #1.

Notice the state, waitforhand, and delay parameters are not provided. That means they will be set to their default values. State will be “on” turning the light on, waitforhand will be “true” meaning P2L will wait for a hand, and delay will be “300”.

Here is the XML file used in the screenshots of this document. It contains plenty of useful comments as well.

https://schneiderelectric.sharepoint.com/:u:/r/sites/PortlandSPSGroup/Shared%20Documents/Engineering/Test%20%26%20Automation%20Engineering/Pick%20To%20Light%202/Example%20XML%20File.xml?csf=1&web=1&e=aq93bT